| # DRINKING WATER |  | NON PACKAGED |  |  | PACKAGED |
| --- | --- | --- | --- | --- | --- |
| SR NO. | # IMPURITY | # IS # Desirable limit # (As per IS 10500 : 2012) | USEPA Maximum Contaminant level As per EPA -,2012 | W. H. O. Guideline valueAs per Vol 1,Recommendation 2011 ) | # IS # Desirable # Limit IS 14543 |
|  | Colour Hazen units | 5 | NA | 15 true colour units | 2 |
|  | Odour | Unobjecti- Onable | 3 threshold odour number | Not offensive | Agreeable |
|  | Taste | Agreeable | NA | Not offensive | Agreeable |
|  | Turbidity NTU | 5 | 5 | 5 | 2 |
|  | Dissolved solids mg/l | 500 | 500 | 1000 | 500 |
|  | Total hardness mg/l\ Calcium mg/l Magnesium mg/l | 300 75 --- | NA --- --- | 500 (as CaCo3) --- --- | NA 75 30 |
|  | pH value | 6.5 - 8.5 | 6.5-8.5 | 6.5-8.5 | 6.5 – 8.5 |
|  | Copper mg/l | 0.05 | 1.3 | 1.0 | 0.05 |
|  | Iron mg/l | 0.3 | 0.3 | 0.3 | 0.1 |
|  | Manganese mg/l | 0.1 | 0.05 | 0.1 | 0.1 |
|  | Chlorides mg/l | 250 | 250 | 250 | 200 |
|  | Sulphate mg/l | 200 | 250 | 400 | 200 |
|  | Nitrate mg/l | 45 | 10 | 10 | 45 |
|  | Fluoride mg/l | 1.0 | 2.0 | 1.5 | 1.0 |
|  | Phenolic compounds | 0.001 | 0.001 | 0.001 | Absent |
|  | Mercury mg/l | 0.001 | 0.002 | 0.001 | 0.001 |
|  | Cadmium mg/l | 0.01 | 0.005 | 0.003 | 0.01 |
|  | Selenium mg/l | 0.01 | 0.05 | 0.01 | 0.01 |
|  | Arsenic mg/l | 0.05 | 0.01 | 0.01 | 0.05 |
|  | Cyanide mg/l | 0.05 | 0.2 | 0.1 | Absent |
|  | Lead mg/l | 0.05 | 0.015 | 0.01 | 0.01 |
|  | Zinc mg/l | 5 | 5 | 3 | 5 |
|  | Anionic detergents | 0.2 | - | 0.2 | 0.2 |
|  | Total pesticides mg/l | NIL | 0.0005 | 0.0005 | 0.0005 |
|  | Total bacterial count C.F.U / ml | - | Not more than 500 |  | 100/ml at 200C 20/ml at 370C |
|  | Total coliform per 100ml(including fecal coliform& E. coli) | Absent | Absent | Absent | Absent |
|  | Viruses (Enteric) | Absent | Absent | Absent | Absent |

DRINKING WATER STANDARDS